Simulator Testbed for UAV Swarm Path Planning and Control

Progress Report — Foundations of Robotics

# Team Details

Group Number: 10

| S.No. | Name | Roll Number |
| --- | --- | --- |
| 1 | Samandeep | IIT2023077 |
| 2 | Ankit | IIT2023120 |
| 3 | Shraddha | IIT2023114 |
| 4 | Divyansh | IIT2023111 |
| 5 | Kinshuk | IIT2023079 |
| 6 | Tejanshu | MRM2025014 |

# Problem Statement

Unmanned Aerial Vehicle (UAV) swarms are increasingly being used for tasks such as surveillance, search-and-rescue, agricultural monitoring, and disaster response, where a group of drones must operate together in a shared, often unknown, environment. Coordinating such swarms is difficult: individual UAVs must plan collision-free paths in real time, adapt to obstacles and no-fly zones, maintain formation or divide mission roles amongst themselves, and do all of this despite limited onboard compute and constrained inter-agent communication. Testing such algorithms directly on real hardware is expensive, unsafe at scale, and slow to iterate on, which motivates the need for a reliable, lightweight simulation testbed.

This project aims to build a simulator testbed for UAV swarm path planning and control that supports simultaneous path planning, collision avoidance, communication-constrained coordination, formation/coordinated control, and mission allocation across multiple UAVs. The testbed will incorporate map-based mission specification (including obstacle maps and no-fly zones), realistic UAV flight dynamics, and multi-agent reinforcement learning (MARL) as the core decision-making paradigm, supporting both Centralized-Training-with-Decentralized-Execution (CTDE) and fully decentralized policy architectures. Trained policies will be evaluated on metrics that matter for real deployment: path length, area coverage, collision rate, mission-completion rate, energy consumption, communication overhead, and scalability as swarm size grows.

The end goal is a reusable, extensible platform — not a one-off demo — on which different swarm coordination algorithms can be trained, compared, and benchmarked fairly under identical simulated conditions, giving the team a controlled way to study the trade-offs between centralized and decentralized swarm intelligence before any real-world deployment is attempted.

# Literature Review

Our review focused on three interlinked strands of prior work: (i) existing UAV swarm simulators, (ii) multi-agent path-planning and control approaches, and (iii) reinforcement learning methods used for swarm coordination. A recurring theme across all three is a trade-off between physical realism, multi-agent scalability, and how fast a simulator can generate the millions of environment steps that modern RL training needs — which is directly relevant to the simulator backend our team has chosen.

Existing UAV swarm simulators — the PyBullet lineage

The most widely adopted starting point in this space is gym-pybullet-drones (Panerati et al., IROS 2021), a PyBullet-based Gym/Gymnasium environment for the Bitcraze Crazyflie 2.x that models ground effect, blade drag, and inter-drone downwash, and provides single- and multi-agent training scripts. It has since become the de-facto baseline that a large body of follow-up work compares itself against, including safe-control-gym (Yuan et al., 2022), which targets safe learning-based control on a similar PyBullet Crazyflie model, and CrazySwarm2, which focuses on ROS 2 firmware-in-the-loop and real-hardware integration. These PyBullet-based tools are easy to install, well documented, and physically reasonable, but they run on a CPU-bound Python physics loop with no native GPU vectorization — so training a decentralized MARL policy across a large swarm, or sweeping the swarm-size-scalability metric our project needs, becomes a throughput bottleneck rather than a modelling one.

Existing UAV swarm simulators — the MuJoCo / GPU-vectorized lineage

A newer generation of simulators addresses exactly this bottleneck. MuJoCo-Drones-Gym (Tayal, 2026) ports the gym-pybullet-drones design philosophy onto the MuJoCo physics engine, keeping the same aerodynamic models, drone parameters, and PID architecture so that existing controllers and policies transfer with minimal change, while adding a PettingZoo ParallelEnv wrapper for native MARL, procedural obstacle generators (forest/urban/indoor/random/gates), wind and turbulence models, domain randomization, and — most importantly for us — a GPU-vectorized MJX backend built on JAX that can run thousands of parallel environments in a single accelerator call. Related efforts push the same GPU-first idea further: Aerial Gym Simulator (Kulkarni et al., IEEE RA-L 2025) uses NVIDIA Isaac Gym with parallelized geometric controllers reported to scale to 100,000+ simultaneous drones, OmniDrones (Xu et al., 2024) does something similar on Isaac Sim/Isaac Lab, and Crazyflow (2026) offers a fully differentiable JAX + MuJoCo-MJX simulator aimed at massively-batched swarm RL and sim-to-real transfer. Weighing the two lineages against our project's requirements — particularly the need to train MARL policies across a growing swarm size without the simulator itself becoming the limiting factor, and our instructor's explicit note that PyBullet has become comparatively slow — the team has decided to build on the MuJoCo lineage, specifically MuJoCo-Drones-Gym, since it does not require ROS, keeps a pip-installable pure-Python workflow, and still exposes task templates (formation flight, multi-agent hover, waypoint navigation) that map closely onto our own requirements.

Multi-agent path planning and control

Classical approaches to multi-UAV path planning — artificial potential fields, RRT*/PRM variants, and Voronoi-based decomposition — remain useful as non-learned baselines and for generating comparison trajectories, but scale poorly with swarm size and struggle with dynamic, partially-observed obstacles. More recent work frames coordination directly as a multi-agent RL problem: Centralized-Training-with-Decentralized-Execution (CTDE) methods such as MAPPO and QMIX let a critic see the global state during training while each UAV acts on local observations at deployment. A 2026 study on communication-aware MARL for decentralized UAV deployment models the swarm as an agent–entity graph with attention-based embeddings, and reports strong coverage results even under restricted communication and partial observability — this is a close conceptual match for the mission-allocation and communication-constraint requirements of our own project.

Reinforcement learning methods for swarm coordination

On the RL-algorithm side, a 2025 comparative study (Drones journal) benchmarks PPO, SAC, DDPG, and TRPO on multi-UAV path planning in both 2D and 3D simulated settings with static obstacles and inter-agent collision constraints, and is a useful reference for which algorithms are worth trying first on our own testbed. Separately, a 2026 energy-aware CTDE framework (E-MAPPO-CTDE) folds a realistic energy-consumption model — based on displacement, acceleration, and flight time — directly into the training objective, which is directly relevant to the energy-consumption metric our project is required to report.

Taken together, the literature suggests a practical path for us: use a MuJoCo-based, GPU-vectorized simulator as the physics/RL backbone (rather than PyBullet, for the speed reasons above), borrow reward and metric design from the comparative RL studies, and borrow the graph/attention-based CTDE architecture idea for the communication-constrained and mission-allocation parts of our own testbed. This literature review will be discussed further in class as instructed.

# Progress and Implementation

Repository setup

A GitHub repository has been created for the project. The four team members whose GitHub usernames were available — Samandeep (Darkknight0125), Ankit (Ankitgarg2105), Kinshuk (Kinshukk098), and Tejanshu (Tejanshu22) — have all been added as collaborators. Shraddha and Divyansh have not yet been added since their GitHub usernames were not available at the time of repository creation; they have been asked to share their usernames so they can be added immediately. The repository will be handed over to the official course organization once this is complete.

Project structure and code implementation

At this stage the repository contains only the initial setup (README and licensing); no project code has been written yet. Over the past few days the team's effort has gone into the literature review above and, in particular, into deciding on a simulation backend — comparing the PyBullet-based approach (gym-pybullet-drones and its derivatives) against the newer MuJoCo-based approach (MuJoCo-Drones-Gym), and selecting MuJoCo-Drones-Gym as our base simulator for the reasons discussed in Section 3. The planned repository structure going forward is:

- docs/ — literature notes, design documents, this report

- src/ — simulator setup, custom obstacle/no-fly-zone maps, mission specification, MARL algorithms (CTDE and decentralized policy implementations)

- readme/ — readme

Experiments

No experiments have been run yet. The immediate next steps for the team are to (1) install MuJoCo-Drones-Gym locally and verify the environment runs correctly, (2) reproduce the existing MultiHoverAviary and PettingZoo-based multi-agent example as a sanity check, and (3) begin extending the obstacle/mission configuration to reflect our own no-fly-zone and mission-allocation requirements before introducing our MARL training pipeline.